# INTENT

# This section covers the construction of storm and sanitary sewage forcemains and appurtenances.

# RELATED WORK SPECIFIED ELSEWHERE

# Section 02530 – Landscaping

# Section 02570 – Excavation and Backfilling of Trenches

# PRODUCTS

# Pipe - High density polyethylene pipe complying with OPSS 1842 and of the size and class shown on the drawings.

# Gates Valves – AWWA C-509 Non-rising spindle, mechanical joint ‘O’ ring seal.

# Air Relief Valves – Vent-O-Mat 50RBX, 50 mm in diameter.  (Edit if combined vacuum relief is required.)

# Bedding – Compactable granular material with no stones greater than 25 mm in diameter.

# Granular Backfill – Granular B complying with OPSS 1010.

# EXECUTION

# OPSS 412, except that leakage allowances will comply with OPSS 701.

# BASIS OF PAYMENT / MEASUREMENT FOR PAYMENT

# OPSS 412, except that the unit price bid for this item shall include the cost of all restoration and connections to structures.